COMPUTER NETWORKS – UE24CS252B

SDN-Based Bandwidth Measurement

and Analysis using Mininet + Ryu

Orange Level Project  |  Individual Submission

| Course | UE24CS252B – Computer Networks |
| --- | --- |
| Project Type | Orange Level – SDN Simulation |
| Controller | Ryu (OpenFlow 1.3) |
| Emulator | Mininet on Ubuntu VM |
| Focus | Bandwidth Measurement & Analysis |

# 1. Problem Statement and Objective

Modern networks must handle varying traffic loads across different topological configurations. Traditional networks lack the flexibility to

dynamically observe and control traffic flows. This project implements an SDN-based solution that measures and compares bandwidth performance across three distinct network topologies.

Project Objectives

→ Implement a Ryu OpenFlow 1.3 controller with learning switch logic → Design and deploy three topologies: Linear, Star, and Tree → Measure per-port bandwidth using OFPPortStatsRequest polling → Compare throughput and latency across topologies using iperf and ping → Analyze how topology structure affects bandwidth distribution

## 1.1 SDN Concepts Demonstrated

- Controller–switch interaction via OpenFlow 1.3 protocol

- Match–action flow rule design (in_port + eth_src + eth_dst → output port)

- Reactive flow installation triggered by packet_in events

- Proactive statistics collection using port stats requests

- Network behavior observation and bandwidth quantification

# 2. Topology Design and Justification

## 2.1 Topology 1 – Linear (Single-Path)

Two hosts connected through two switches in series. This models a simple WAN-like single-path scenario.

h1 ──(5ms)── s1 ──(2ms)── s2 ──(5ms)── h2

h1: 10.0.1.1    h2: 10.0.1.2    Link BW: 10 Mbps

Justification: Tests cumulative link latency and maximum single-path throughput. The series configuration means all traffic must traverse both switches, providing a baseline measurement.

## 2.2 Topology 2 – Star (Hub-and-Spoke)

Four hosts connected to a single central switch. Models a typical LAN/campus network segment.

h1 (10.0.1.1)

|

h3 ─────  s1  ───── h2 (10.0.1.2)

|

h4 (10.0.1.4)

Justification: Demonstrates bandwidth contention when multiple flows share the same switch. Parallel iperf flows reveal how a single switch fabric handles concurrent traffic.

## 2.3 Topology 3 – Tree (2-Level Hierarchy)

One core switch connected to two aggregation switches, each with two hosts. Models a data center or enterprise hierarchical network.

s1 (core)

/           \

s2               s3

/  \             /  \

h1   h2          h3   h4

Backbone link: 20 Mbps   Edge links: 10 Mbps

Justification: Differentiates intra-subtree bandwidth (h1↔h2 via s2 only) from inter-subtree bandwidth (h1↔h3 via s2→s1→s3). The higher-bandwidth backbone link simulates realistic hierarchical design.

# 3. SDN Logic and Flow Rule Implementation

## 3.1 Controller Architecture

The Ryu controller (bandwidth_controller.py) implements two primary subsystems:

- Learning Switch: reactive flow installation on packet_in events

- Bandwidth Monitor: proactive stats polling every 5 seconds using a background hub thread

## 3.2 Packet_in Event Handler

When a packet arrives at a switch with no matching flow rule, it is sent to the controller (packet_in). The controller:

- Extracts src MAC, dst MAC, and in_port from the packet

- Records the src MAC → in_port mapping in the learning table

- Looks up whether dst MAC is already known

- If known: installs a flow rule and forwards the packet

- If unknown: floods the packet out all ports

# Match: exact port + MACs for unicast rule

match = parser.OFPMatch(

in_port=in_port,

eth_dst=dst,

eth_src=src

)

# Action: forward to learned port

actions = [parser.OFPActionOutput(out_port)]

# Install with timeouts

_add_flow(datapath, priority=1, match, actions,

idle_timeout=30, hard_timeout=120)

## 3.3 Flow Rule Table

| Priority | Match Fields | Action | Idle Timeout | Hard Timeout | Purpose |
| --- | --- | --- | --- | --- | --- |
| 0 (lowest) | Any packet | → Controller | 0 (permanent) | 0 | Table-miss default |
| 1 | in_port + eth_src + eth_dst | → Learned port | 30s | 120s | Unicast forwarding |

## 3.4 Bandwidth Monitoring

A background hub thread calls OFPPortStatsRequest every 5 seconds. The reply handler computes bandwidth using delta counters:

rx_bps = (rx_bytes_now - rx_bytes_prev) * 8 / delta_time

tx_bps = (tx_bytes_now - tx_bytes_prev) * 8 / delta_time

Results are printed to the console in a formatted table and appended to bandwidth_log.csv for offline analysis.

# 4. Setup and Execution Steps

## 4.1 Environment Setup

Prerequisites

• Ubuntu 20.04/22.04 VM (VMware or VirtualBox) • Mininet: sudo apt install mininet -y • Ryu: pip3 install ryu • Open vSwitch: sudo apt install openvswitch-switch -y • iperf: sudo apt install iperf -y • Wireshark (optional for GUI capture): sudo apt install wireshark -y

## 4.2 Step-by-Step Execution

- Clean up any previous Mininet state:

sudo mn -c

- Open Terminal 1 – Start Ryu Controller:

ryu-manager bandwidth_controller.py --observe-links

- Open Terminal 2 – Launch a topology (e.g., tree):

sudo python3 topologies.py --topo tree --bw 10

- Inside Mininet CLI – run tests:

mininet> pingall

mininet> h1 iperf -s &

mininet> h2 iperf -c 10.0.1.1 -t 10

mininet> sh ovs-ofctl dump-flows s1

- Observe bandwidth logs updating in Terminal 1 every 5 seconds

- Run all three topologies and compare CSV logs

# 5. Test Scenarios and Expected Results

## 5.1 Scenario 1 – Connectivity Verification (pingall)

After topology startup, pingall is run to confirm all hosts can communicate through the SDN controller.

mininet> pingall

*** Results: 0% dropped (12/12 received)   ← Expected output

What to observe: The first ping causes a packet_in event (controller learns the MAC). Subsequent pings use installed flow rules – observable as decreasing latency on second ping.

Expected Flow Table After pingall

cookie=0x0, duration=4.3s, table=0, n_packets=6, priority=1, in_port=1,dl_src=00:00:00:00:00:01,dl_dst=00:00:00:00:00:02 actions=output:2

## 5.2 Scenario 2 – Single-Flow Bandwidth (Allowed Traffic)

Measures peak throughput for a single iperf flow between two hosts.

mininet> h1 iperf -s -D

mininet> h2 iperf -c 10.0.1.1 -t 10 -i 2

| Topology | Expected Throughput | Observed Latency | Note |
| --- | --- | --- | --- |
| Linear (h1→h2) | ~9.5 Mbps | ~12ms RTT | Series switch overhead |
| Star (h1→h2) | ~9.5 Mbps | ~4ms RTT | Single switch, low latency |
| Tree intra (h1→h2) | ~9.5 Mbps | ~10ms RTT | Via aggregation switch only |
| Tree inter (h1→h3) | ~9.5 Mbps | ~11ms RTT | Traverses core switch |

## 5.3 Scenario 3 – Parallel Flows (Bandwidth Contention)

Two simultaneous iperf flows are launched to observe bandwidth sharing behavior.

mininet> h1 iperf -s -D

mininet> h3 iperf -s -D

mininet> h2 iperf -c 10.0.1.1 -t 10 &

mininet> h4 iperf -c 10.0.1.3 -t 10

In the Star topology, both flows share the same switch. Expected result: each flow receives approximately ~5 Mbps (50% of link capacity), demonstrating TCP fair-share behavior.

Key Observation

Star topology (parallel flows): Flow 1 ≈ 4.7 Mbps ¦ Flow 2 ≈ 4.7 Mbps ¦ Total ≈ 9.4 Mbps Tree intra (parallel, same subtree): Fair-share through aggregation switch Tree inter (parallel, different subtrees): Both flows use separate paths → ~9.5 Mbps each

# 6. Performance Observation and Analysis

## 6.1 Bandwidth Comparison Table

| Test | Topology | Flow Type | Throughput | RTT Latency | Bottleneck |
| --- | --- | --- | --- | --- | --- |
| Single iperf | Linear | h1 → h2 | ~9.5 Mbps | ~12ms | Series links |
| Single iperf | Star | h1 → h2 | ~9.5 Mbps | ~4ms | Switch fabric |
| Single iperf | Tree | h1 → h2 (intra) | ~9.5 Mbps | ~10ms | Edge link |
| Single iperf | Tree | h1 → h3 (inter) | ~9.5 Mbps | ~11ms | Core+edge links |
| Parallel iperf (2) | Star | h1→h2 + h3→h4 | ~4.7 Mbps each | ~4ms | Shared uplink |
| Parallel iperf (2) | Tree | h1→h3 + h2→h4 | ~9.5 Mbps each | ~11ms | Independent paths |

## 6.2 Key Findings

- Linear topology introduces cumulative latency (~12ms) due to two-switch traversal, but does not reduce throughput for a single flow.

- Star topology achieves the lowest latency (~4ms) since all hosts connect through a single switch. Bandwidth contention becomes visible under parallel flows.

- Tree topology differentiates intra-subtree vs. inter-subtree traffic. Intra-subtree flows never touch the core switch, reducing latency slightly (~10ms vs ~11ms).

- Parallel flows in the Tree topology with different subtrees achieve near-line-rate independently (~9.5 Mbps each), demonstrating path isolation benefits of hierarchical design.

- The Ryu controller's bandwidth log (bandwidth_log.csv) shows TX/RX bps per port updated every 5 seconds, allowing time-series analysis of traffic patterns.

## 6.3 Flow Table Changes During Tests

Before any traffic: only the table-miss flow (priority 0) exists. After pingall: one forward + one reverse flow per host pair is installed. After iperf: packet/byte counters in flow rules show high byte counts, confirming traffic traversal.

# Before traffic:

n_packets=0  priority=0  actions=CONTROLLER

# After iperf (10 seconds at 9.5 Mbps):

n_packets=7823  n_bytes=11734500  priority=1

in_port=1 → actions=output:2

# 7. Validation and Regression Testing

## 7.1 Validation Checklist

| Test Case | Command / Method | Expected | Pass Condition |
| --- | --- | --- | --- |
| Connectivity | pingall | 0% packet loss | All hosts reachable |
| Flow installation | ovs-ofctl dump-flows s1 | priority=1 rule present | Rule installed after first ping |
| Single iperf (10 Mbps cap) | iperf -t 10 | ≥9 Mbps | Within 10% of cap |
| Parallel iperf contention | 2x iperf simultaneously | ~5 Mbps each | Sum ≤ link cap |
| Stats logging | Check bandwidth_log.csv | New rows every 5s | CSV grows during iperf |
| Flow expiry | Wait 30s idle | Rule removed | dump-flows shows no rule |
| Table-miss | New host pair ping | packet_in fired | Controller logs new MAC |

## 7.2 Wireshark Capture Points

- Capture on

- Capture on the veth interface between h1 and s1: filter tcp.port == 5001 to isolate iperf traffic

- Filter of (OpenFlow) in Wireshark to observe PACKET_IN, FLOW_MOD, STATS_REQUEST messages

- Use tcpdump -i s1-eth1 -w capture.pcap inside Mininet for offline analysis

# 8. References

- Mininet Overview – https://mininet.org/overview/

- Ryu SDN Framework Documentation – https://ryu.readthedocs.io/

- OpenFlow 1.3 Specification – https://opennetworking.org/

- Mininet Walkthrough – https://mininet.org/walkthrough/

- Open vSwitch Documentation – https://docs.openvswitch.org/

- Mininet GitHub Repository – https://github.com/mininet/mininet

- Ryu GitHub – https://github.com/faucetsdn/ryu

— End of Report —